H5SDK Documentation

2021-09

目录

1. SDK description	3

1.1. Document Description	3

2. Device communication interface	3

2.1. Device communication interface description	3

2.1.1. Device communication mode	3

2.1.2. Device connection mechanism	3

2.1.3. Device connection process	4

2.1.4. Dictionary values	4

2.2. Callback Definition	5

2.2.1. Connection Callback	5

2.2.2. Login callback	6

2.2.3. Connection disconnection callback	7

2.2.4. Open stream callback	8

2.2.5. PTZ control callback	9

2.2.6. Real-time video stream callback	9

2.2.7. Playback video stream callback	11

2.2.8. Query playback record callback	12

2.2.9. Playback retrieval end callback	13

2.2.10. p2p error callback	14

2.3. External call method	15

2.3.1. Connecting the device	15

2.3.2. Log in device	16

2.3.3. Open stream	17

2.3.4. Close the stream	18

2.3.5. PTZ Control	18

2.3.6. Switch stream	20

2.3.7. Disconnect the device	21

2.3.8. Playback retrieval	21

2.3.9. Stop record retrieval	22

2.3.10. Start playback record	23

2.3.11. Pause playback	24

2.3.12. Continue playback	25

2.3.13. Stop playback	26

2.3.14. Player initialization	26

3. Precautions	27

# SDK description

## Document Description

1. Connector.js, Decoder.js, libde265.js, gutils.js are the main libraries of the SDK

2. The play.js and index.html files are demo demonstrations, which encapsulate some basic logic for secondary development.

3.logo_bg.png is the video background image, you can replace it with other png images with the same name

# Device communication interface

The device communication interface is an agreement with the device communication interface, which is used to connect the device, play video, operate the device PTZ, etc.

## Device communication interface description

### Device communication mode

The SDK uses websockets to communicate with the device and receive video streams, currently using the WSS:// protocol,

When the SDK sends a command to the device, the device will return response information. To receive the response information, the callback function of each command needs to be defined in advance.

### Device connection mechanism

When connecting the device, you need to connect the device first, and then log in to the device after the connection is successful, in order to perform operations such as video playback, video retrieval, and PTZ control.

### Device connection process

| 名称 | Name Description |
| --- | --- |
| Device ID | Unique ID for each device |
| Device IP | The IP of the device in the LAN, the device can be connected through the IP LAN |
| Device Port | Device Port The port number of the device when using IP connection, the default is 10000 |
| Device Username | The username required to log in to the device, the default is admin |
| Device Password | The password required to log in to the device, the default is empty |
| Channel | The number of channels of the device, starting from 0, a single-channel device, that is, the camera only has channel 0 |

### Dictionary values

| encoding mode:enc | H264 H265 |
| --- | --- |
| frame type:frametype | 0:audio frame 1: I frame 2 :I frame |
| Stream type:streamid | 0:Main stream 1:substream |
| fps | 1-25 |
| P2P connection code | 0 success -2 timeout -10 The local connection is closed -11 The remote connection was closed -12 addressing failed -13 Connection failed (device offline) -20 Login device verification failed (wrong password) |
| PTZ Command Type | 0: Stop 1: Auto Pan 2: Up 3: Down 4: Left 5: Right 6: Aperture Plus 7: Aperture Minus 8: Zoom Plus 9: Zoom Minus 10: Focal Length Plus 11: Focal Length Minus 12: Auxiliary switch 13: Set preset 14: Call preset 15: Clear preset |
| Recording type | Timing: 1 Motion: 2 Alarm: 4 Manual: 8 All types: 15, 15 is used when retrieving recordings |

## Callback Definition

### Connection Callback

#### Method description

| No. | Method name | Method description | Calling object |
| --- | --- | --- | --- |
| 1 | onconnect | device connection callback, the response callback after calling the connection method | ConnectApi |

#### Callback Parameters

##### Callback example

ConnectApi.onconnect = function (session, code) { if (code === 0) { ConnectApi.login(session, session.user, session.pwd); } };

##### Parameter description

| No. | Parameter name | Parameter type | Description |
| --- | --- | --- | --- |
| 1 | session | Object | connection object |
| 2 | code | Number | Number Status code, to judge whether the connection is successful or failed, 0 is success, others are failure, the failure may be that the device is offline |

### Login callback

#### Method description

| No. | Method name | Method description | Calling object |
| --- | --- | --- | --- |
| 1 | onloginresult | Device login callback, the response callback ConnectApi after calling the login method | ConnectApi |

#### Callback Parameters

##### Callback example

ConnectApi.onloginresult= function (session, code) { };

##### Parameter description

| No. | Parameter name | Parameter type | Description |
| --- | --- | --- | --- |
| 1 | session | Object | connection object |
| 2 | code | Number | Number Status code, to judge whether the login is successful or unsuccessful, 0 is success, other is failure, failure means the user name or password is wrong |

### Connection disconnection callback

#### Method description

| No. | Method name | Method description | Calling object |
| --- | --- | --- | --- |
| 1 | ondisconnect | The callback when the device connection is disconnected, the callback triggered when the device is passively disconnected due to network and other reasons | ConnectApi |

#### Callback Parameters

##### Callback example

ConnectApi.ondisconnect= function (session, code) { };

##### Parameter description

| No. | Parameter name | Parameter type | Description |
| --- | --- | --- | --- |
| 1 | session | Object | connection object |
| 2 | code | Number | Status code |

### Open stream callback

#### Method description

| No. | Method name | Method description | Calling object |
| --- | --- | --- | --- |
| 1 | onopenstream | Open the device stream callback, call the response callback ConnectApi after calling the open stream method | ConnectApi |

#### Callback Parameters

##### Callback example

ConnectApi.onopenstream= function (session, channel, streamid, result, cam_desc) { };

##### Parameter description

| No. | Parameter name | Parameter type | Description |
| --- | --- | --- | --- |
| 1 | session | Object | connection object |
| 2 | channel | Number | Channel, open the channel corresponding to the stream |
| 3 | streamid | Number | Stream value |
| 4 | code | Number | Status code, 0 is success, others are failure |

### PTZ control callback

#### Method description

| No. | Method name | Method description | Calling object |
| --- | --- | --- | --- |
| 1 | onptzresult | Callback after sending PTZ command | ConnectApi |

#### Callback Parameters

##### Callback example

ConnectApi.onptzresult= function (session, code) { };

##### Parameter description

| No. | Parameter name | Parameter type | Description |
| --- | --- | --- | --- |
| 1 | session | Object | connection object |
| 2 | code | Number | Status code, 0 is success, others are failure |

### Real-time video stream callback

#### Method description

| No. | Method name | Method description | Calling object |
| --- | --- | --- | --- |
| 1 | onrecvframeex | Real-time video stream callback | ConnectApi |

#### Callback Parameters

##### Callback example

ConnectApi.onrecvframeex=function (session, frametype, data, datalen, channel, width, height, enc, fps, timestamp) { };

##### Parameter description

| No. | Parameter name | Parameter type | Description |
| --- | --- | --- | --- |
| 1 | session | Object | connection object |
| 2 | frametype | Number | frame type 0 audio frame 1 I frame 2 P frame |
| 3 | data | Buffer | video frame data |
| 4 | datalen | Number | Video data length |
| 5 | channel | Number | channel value |
| 6 | width | Number | screen width |
| 7 | height | Number | screen height |
| 8 | enc | String | Encoding format |
| 9 | fps | Number | frame rate |
| 10 | timestamp | Number | Timestamp of this frame |

### Playback video stream callback

#### Method description

| No. | Method name | Method description | Calling object |
| --- | --- | --- | --- |
| 1 | onrecvrecframe | The callback method after sending the open playback command | ConnectApi |

#### Callback Parameters

##### Callback example

ConnectApi.onrecvrecframe = function (session, frametype, data, datalen, channel, width, height, enc, fps, ts_ms)){ }

##### Parameter description

| No. | Parameter name | Parameter type | Description |
| --- | --- | --- | --- |
| 1 | session | Object | connection object |
| 2 | frametype | Number | frame type 0 audio frame 1 I frame 2 P frame |
| 3 | data | Buffer | video frame data |
| 4 | datalen | Number | Video data length |
| 5 | channel | Number | channel value |
| 6 | width | Number | screen width |
| 7 | height | Number | screen height |
| 8 | enc | String | Encoding format |
| 9 | fps | Number | frame rate |
| 10 | timestamp | Number | Timestamp of this frame |

### Query playback record callback

#### Method description

| No. | Method name | Method description | Calling object |
| --- | --- | --- | --- |
| 1 | onsearchrec | Callback method after sending playback retrieval command | ConnectApi |

#### Callback Parameters

##### Callback example

ConnectApi.onsearchrec = function (session, channel, file_type, file_begintime, file_endtime, file_total){ }

##### Parameter description

| No. | Parameter name | Parameter type | Description |
| --- | --- | --- | --- |
| 1 | session | Object | connection object |
| 2 | channel | Number | channel value |
| 3 | file_type | Number | Recording type, Timing: 1 Movement: 2 Alarm: 4 Manual: 8 |
| 4 | file_begintime | Number | Recording start time |
| 5 | file_endtime | String | Recording end time |
| 6 | file_total | Number | Retrieve the total number of recordings |

### Playback retrieval end callback

#### Method description

| No. | Method name | Method description | Calling object |
| --- | --- | --- | --- |
| 1 | onsearchrecend | The callback method triggered after the playback video is retrieved | ConnectApi |

#### Callback Parameters

##### Callback example

ConnectApi.onsearchrecend= function (session){ }

##### Parameter description

| No. | Parameter name | Parameter type | Description |
| --- | --- | --- | --- |
| 1 | session | Object | connection object |

### p2p error callback

#### Method description

| No. | Method name | Method description | Calling object |
| --- | --- | --- | --- |
| 1 | onp2perror | P2P connection error callback | ConnectApi |

#### Callback Parameters

##### Callback example

ConnectApi.onp2perror= function (session, code){ }

##### Parameter description

| No. | Parameter name | Parameter type | Description |
| --- | --- | --- | --- |
| 1 | session | Object | connection object |
| 2 | code | Number | Error code |

## External call method

### Connecting the device

#### Method description

| No. | Method name | Method description | Calling object |
| --- | --- | --- | --- |
| 1 | ConnectDevice | Connect device method, send connect device command | Player |

#### Call parameters

##### Call example

Player.ConnectDevice(deviceid, ip, user, pwd, winindex, port, connectType, channel, streamid)

##### Parameter description

| No. | Parameter name | Parameter type | Description |
| --- | --- | --- | --- |
| 1 | deviceid | String | Device ID |
| 2 | ip | String | Device IP, use the device ID to connect first, use the IP connection when there is no device ID, and allow it to be empty when there is an ID |
| 3 | user | String | device username |
| 4 | pwd | String | device password |
| 5 | winindex | Number | join index |
| 6 | port | Number | It is used when connecting by IP, it is recommended to use port 10000 |
| 7 | connectType | Number | Connection mode 0: Pre-connect 1: Connect and open stream |
| 8 | channel | Number | channel value |
| 9 | streamid | Number | Stream type, valid when the connection mode is 1 |

### Log in device

#### Method description

| No. | Method name | Method description | Calling object |
| --- | --- | --- | --- |
| 1 | login | The command sent after the connection is successful, log in to the device | ConnectApi |

#### Call parameters

##### Call example

ConnectApi.login(session, username, password);

##### Parameter description

| No. | Parameter name | Parameter type | Description |
| --- | --- | --- | --- |
| 1 | session | Object | connection object |
| 2 | username | String | device username |
| 3 | password | String | device password |

### Open stream

#### Method description

| No. | Method name | Method description | Calling object |
| --- | --- | --- | --- |
| 1 | OpenStream | Turning on the device stream will trigger the real-time video stream callback | Player |

#### Call parameters

##### Call example

Player.OpenStream(deviceid, ip, channel, streamid, winindex)

##### Parameter description

| No. | Parameter name | Parameter type | Description |
| --- | --- | --- | --- |
| 1 | deviceid | String | Device ID |
| 2 | ip | String | Device IP |
| 3 | channel | Number | channel value |
| 4 | streamid | Number | Stream type |
| 5 | winindex | Number | Rendered window index |

### Close the stream

#### Method description

| No. | Method name | Method description | Calling object |
| --- | --- | --- | --- |
| 1 | CloseStream | Close device stream | Player |

#### Call parameters

##### Call example

Player.CloseStream(keyindex)

##### Parameter description

| No. | Parameter name | Parameter type | Description |
| --- | --- | --- | --- |
| 1 | keyindex | Number | Vidoe window index |

### PTZ Control

#### Method description

| No. | Method name | Method description | Calling object |
| --- | --- | --- | --- |
| 1 | ptz_ctrl | Control device PTZ operation | Player |

#### Call parameters

##### Call example

Player.ptz_ctrl(deviceid, ip, channel, type, param)

##### Parameter description

| No. | Parameter name | Parameter type | Description |
| --- | --- | --- | --- |
| 1 | deviceid | String | Device ID |
| 2 | ip | String | Device IP |
| 3 | channel | Number | channel value |
| 4 | type | Number | PTZ action 0: Stop 1: Auto pan 2: Up 3: Down 4: Left 5: Right 6: Aperture add 7: Aperture reduce 8: Zoom add 9: Zoom reduce 10: Focus add 11: Focus reduce 12: Assist Switch 13: Set preset 14: Call preset 15: Clear preset |
| 5 | param | Number | PTZ control parameters, set according to the action, 1-5 represent the speed, 13 and 14 are the preset positions |

### Switch stream

#### Method description

| No. | Method name | Method description | Calling object |
| --- | --- | --- | --- |
| 1 | ChangeStream | Switch device stream | Player |

#### Call parameters

##### Call example

Player.ChangeStream(deviceid, ip, channel, streamid, winindex)

##### Parameter description

| No. | Parameter name | Parameter type | Description |
| --- | --- | --- | --- |
| 1 | deviceid | String | Device ID |
| 2 | ip | String | Device IP |
| 3 | channel | Number | aisle |
| 4 | streamid | Number | To switch the stream type |
| 5 | winindex | Number | Rendered window index |

### Disconnect the device

#### Method description

| No. | Method name | Method description | Calling object |
| --- | --- | --- | --- |
| 1 | DisConnectDevice | Disconnect the device | Player |

#### Call parameters

##### Call example

Player.DisConnectDevice(deviceid, ip)

##### Parameter description

| No. | Parameter name | Parameter type | Description |
| --- | --- | --- | --- |
| 1 | deviceid | String | Device ID |
| 2 | ip | String | Device IP |

### Playback retrieval

#### Method description

| No. | Method name | Method description | Calling object |
| --- | --- | --- | --- |
| 1 | SreachRecord | Query the recording of the specified channel of the device. Note that two executions cannot be performed at the same time. The caller should prevent the user from performing other operations during the query. | Player |

#### Call parameters

##### Call example

Player.SreachRecord(deviceid, ip, channel, begintime, endtime, type)

##### Parameter description

| No. | Parameter name | Parameter type | Description |
| --- | --- | --- | --- |
| 1 | deviceid | String | Device ID |
| 2 | ip | String | Device IP |
| 3 | channel | Number | channel value |
| 4 | begintime | Number | start time timestamp in seconds |
| 5 | endtime | Number | end time timestamp in seconds |
| 6 | type | Number | Recording Type, Timing: 1 Movement: 2 Alarm: 4 Manual: 8 All Types: 15, |

### Stop record retrieval

#### Method description

| No. | Method name | Method description | Calling object |
| --- | --- | --- | --- |
| 1 | Stopsearch | Stop record retrieval | Player |

#### Call parameters

##### Call example

Player.Stopsearch(deviceid, ip)

##### Parameter description

| No. | Parameter name | Parameter type | Description |
| --- | --- | --- | --- |
| 1 | deviceid | String | Device ID |
| 2 | ip | String | Device IP |

### Start playback record

#### Method description

| No. | Method name | Method description | Calling object |
| --- | --- | --- | --- |
| 1 | StartPlayBack | Play back the video at the specified time point | Player |

#### Call parameters

##### Call example

Player.StartPlayBack(deviceid, ip, channel, begintime, endtime, type, winindex, isSound)

##### Parameter description

| No. | Parameter name | Parameter type | Description |
| --- | --- | --- | --- |
| 1 | deviceid | String | Device ID |
| 2 | ip | String | Device IP |
| 3 | channel | Number | channel value |
| 4 | begintime | Number | Recording start time Second-level timestamp |
| 5 | endtime | Number | Recording end time Second-level timestamp |
| 6 | type | Number | Recording Type, Timing: 1 Movement: 2 Alarm: 4 Manual: 8 All Types: 15, |
| 7 | winindex | Number | window index |
| 8 | isSound | Boolean | Whether to play audio, the fields encapsulated by business logic, can not be passed |

### Pause playback

#### Method description

| No. | Method name | Method description | Calling object |
| --- | --- | --- | --- |
| 1 | PausePlayBack | Pause the current playback | Player |

#### Call parameters

##### Call example

Player.PausePlayBack(deviceid, ip)

##### Parameter description

| No. | Parameter name | Parameter type | Description |
| --- | --- | --- | --- |
| 1 | deviceid | String | Device ID |
| 2 | ip | String | Device IP |

### Continue playback

#### Method description

| No. | Method name | Method description | Calling object |
| --- | --- | --- | --- |
| 1 | ContinuePlayBack | Continue current playback | Player |

#### Call parameters

##### Call example

Player.ContinuePlayBack(deviceid, ip)

##### Parameter description

| No. | Parameter name | Parameter type | Description |
| --- | --- | --- | --- |
| 1 | deviceid | String | Device ID |
| 2 | ip | String | Device IP |

### Stop playback

#### Method description

| No. | Method name | Method description | Calling object |
| --- | --- | --- | --- |
| 1 | StopPlayBack | Stop playback | Player |

#### Call parameters

##### Call example

Player.StopPlayBack(deviceid, ip, channel)

##### Parameter description

| No. | Parameter name | Parameter type | Description |
| --- | --- | --- | --- |
| 1 | deviceid | String | Device ID |
| 2 | ip | String | Device IP |
| 3 | channel | Number | channel value |

### Player initialization

#### Method description

| No. | Method name | Method description | Calling object |
| --- | --- | --- | --- |
| 1 | init | Player initialization, pass in an array of canvas elements, such as 4 windows, pass in an array containing these 4 canvas elements | Player |

#### Call parameters

##### Call example

Player.init(playerArr)

##### Parameter description

| No. | Parameter name | Parameter type | Description |
| --- | --- | --- | --- |
| 1 | playerArr | Array | Canvas array |

# Precautions

1. The player background image needs to be named logo_bg.png and placed in the same path as the html file of the interface, and it needs to run on the web server to load the background image

2. Browser decoding performance is limited, please do not play video more than 4 screens at the same time

3. The user name of the device is admin by default, and the password is empty by default. If the device is successfully connected but cannot log in, the password may be wrong. You can try to reset the device and try again.

4.The Canvas tag of the player must specify the width and height. The value cannot be 100%, otherwise the screen will be rendered blurry. If the interface needs to be adaptive, the width and height must be reset after each change.